# TASK-1

TITLE: Design and simulation of Basic ALU using Verilog HDL

OBJECTIVE: To design an ALU performing arithmetic and logical operations

OPERATIONS PERFORMED: 1. Addition

2. Subtraction

3. AND

4. OR

5.NOT

SOFTWARE USED:  vivado

CODE:

VERILOG CODE FOR ALU:

`timescale 1ns / 1ps

//ALU PROGRAM

module task_1(

input [3:0]a,

input [3:0]b,

input [2:0]sel,

output reg [3:0]y

);

always@(*)

begin

case(sel)

3'b000 : y = a + b;  //addition

3'b001 : y = a - b;  //subtraction

3'b010 : y = a & b;  //AND operation

3'b011 : y = a | b;  //OR operation

3'b100 : y = ~a;     //not operation

default: y = 4'b0000;//default case

endcase

end

endmodule

TESTBENCH FOR THAT VERILOG CODE:

`timescale 1ns / 1ps

//TESTBENCH FOR ALU

module task_1_tb(    );

reg [3:0]a;

reg [3:0]b;

reg [3:0]sel;

wire [3:0]y;

task_1 uut(

.a(a),

.b(b),

.sel(sel),

.y(y)

);

initial begin

$monitor("a=%b b=%b sel=%b y=%b ",a,b,sel,y);

a=4'b0100; b=4'b0010; sel=3'b000; #10; //4+2=6(0110)

a=4'b0100; b=4'b0010; sel=3'b001; #10; //4-2=2(0010)

a=4'b0100; b=4'b0010; sel=3'b010; #10; //0(0000)

a=4'b0100; b=4'b0010; sel=3'b011; #10; //6(0110)

a=4'b0100; b=4'b0010; sel=3'b100; #10; //B(1011)

$finish;

end

endmodule

| SEL | OPERATION | OUTPUT |
| --- | --- | --- |
| 000 | a + b | 0110 |
| 001 | a - b | 0010 |
| 010 | a & b | 0000 |
| 011 | a ¦ b | 0110 |
| 100 | ~a | 1011 |

EXPECTED OUTPUT:

PRACTICAL OUTPUT:

RESLUT:

ALU designed and simulated successfully.